ASFVOLT16 REDFISH SERVICE

Redfish URIs and Response

- Health status monitoring of Chassis Modules

a) Get Chassis Details

- ubuntu@voltha-78:~$ curl -k https://10.70.47.56:8888/redfish/v1/Chassis {"@odata.context":"/redfish/v1/$metadata#Chassis","@odata.id":"/redfish/v1/Chassis","@odata.type":"#ChassisCollection.ChassisCollection","Name":"Chassis Collection","Description":"Collection of Chassis","Members@odata.count":1,"Members":[{"@odata.id":"/redfish/v1/Chassis/1"}]}

b) Get Fan Details

ubuntu@voltha-78:~$ curl -k https://10.70.47.56:8888/redfish/v1/Chassis/1/Thermal/ {"@odata.context":"/redfish/v1/$metadata#Thermal.Thermal","@odata.id":"/redfish/v1/Chassis/1/Thermal","@odata.type":"#Thermal.Thermal","Name":"Thermal sensors collection","Description":"Collection of Thermal sensors","SensorNumber":7,"Temperatures":[{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Temperatures/1"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Temperatures/2"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Temperatures/3"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Temperatures/4"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Temperatures/5"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Temperatures/6"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Temperatures/7"}],"Fans":[{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Fans/1"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Fans/2"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Fans/3"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Fans/4"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Fans/5"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Fans/6"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Fans/7"},{"@odata.id":"/redfish/v1/Chassis/1/Thermal/Fans/8"}]}

c) Get Individual Fan Health and rpm

ubuntu@voltha-78:~$ curl -k https://10.70.47.56:8888/redfish/v1/Chassis/1/Thermal/Fans/1 {"Fan":{"@odata.type":"#Thermal.v1_0_0.Thermal","PhysicalContext":null,"ReadingUnits":"rpm","Reading":13700,"Id":1,"Status":{"State":"Enabled","Health":"OK"}},"@odata.id":"/redfish/v1/Chassis/1/Thermal/Fans/1"}

- d) Get PSU Details

ubuntu@voltha-78:~$ curl -k https://10.70.47.56:8888/redfish/v1/Chassis/1/Power/PowerSupplies {"@odata.context":"/redfish/v1/$metadata#Power.v1_0_0.PowerSupply","@odata.id":"/redfish/v1/Chassis/1/Power/PowerSupplies","@odata.type":"#Power.v1_0_0.PowerSupply","Name":"Power supply Collection","Description":"Collection of Power supply sensors","Power":[{"@odata.id":"/redfish/v1/Chassis/1/Power/PowerSupplies/1"},{"@odata.id":"/redfish/v1/Chassis/1/Power/PowerSupplies/2"}]}

- e) Get Individual PSU Health and Power output

ubuntu@voltha-78:~$ curl -k https://10.70.47.56:8888/redfish/v1/Chassis/1/Power/PowerSupplies/2 {"PowerSupply":{"@odata.type":"Power.v1_0_0.PowerSupply","PowerConsumedWatts":187,"PsuTemperature":19,"PsuFanRpm":9.6,"MemberId":2,"Status":{"Health":"OK","State":"Enabled"}},"@odata.id":"/redfish/v1/Chassis/1/Power/PowerSupplies/2"}

- c) Submitting PSU alarms to kafka

ubuntu@voltha-78:~kafkacat -C -b localhost:33703 -t voltha.alarms {"category": "OLT", "reported_ts": 1513688448.0, "severity": "WARNING", "resource_id": "olt", "id": "voltha.asfvolt16_olt.000139a9a786d85f.olt", "state": "RAISED", "context": {"Health-check-failed-for-Psu": "2"}, "type": "EQUIPMENT", "raised_ts": 1513688448.0, "changed_ts": 0.0, "description": "OLT Alarm - Health Monitoring asfvolt16 olt - Raised"}

- Health status monitoring of Ethernet Ports

- Ethernet Ports

ubuntu@voltha-78:~$ curl -k https://10.70.47.56:8888/redfish/v1/EthernetSwitches/2/Ports {"@odata.context":"/redfish/v1/$metadata#EthernetSwitches/Members/2/Ports","@odata.id":"/redfish/v1/EthernetSwitches/2/Ports","@odata.type":"#EthernetSwitchPortCollection.EthernetSwitchPortCollection","Name":"Ethernet Switch Port Collection","Description":"Collection of Ethernet Switch Ports","Members@odata.count":20,"Members":[{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/1"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/2"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/3"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/4"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/5"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/6"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/7"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/8"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/9"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/10"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/11"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/12"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/13"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/14"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/15"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/16"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/17"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/18"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/19"},{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/20"}]}

- b) SFP Port

ubuntu@voltha-78:~$ curl -k https://10.70.47.56:8888/redfish/v1/EthernetSwitches/2/Ports/2 {"@odata.context":"/redfish/v1/$metadata#EthernetSwitches/Members/2/Ports/Members/$entity","@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/2","@odata.type":"#EthernetSwitchPort.v1_0_0.EthernetSwitchPort","Id":"2","Name":"Port2","Description":"Ethernet Switch Port description","PortId":"Port ID","BiasCurrent":"[0000]","Status":{"State":"Enabled","Health":"Critical","HealthRollup":"N/A"},"VLANs":{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/2/VLANs"},"Links":{"Switch":{"@odata.id":"/redfish/v1/EthernetSwitches/2"},"PrimaryVLAN":null,"MemberOfPort":null,"PortMembers":[]}}

- c) QSFP Port

- ubuntu@voltha-78:~$ curl -k https://10.70.47.56:8888/redfish/v1/EthernetSwitches/2/Ports/18 {"@odata.context":"/redfish/v1/$metadata#EthernetSwitches/Members/2/Ports/Members/$entity","@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/18","@odata.type":"#EthernetSwitchPort.v1_0_0.EthernetSwitchPort","Id":"18","Name":"Port18","Description":"Ethernet Switch Port description","PortId":"Port ID","BiasCurrent":"[75e4, 756a, 75e4, 7c92]","Status":{"State":"Enabled","Health":"OK","HealthRollup":"OK"},"VLANs":{"@odata.id":"/redfish/v1/EthernetSwitches/2/Ports/18/VLANs"},"Links":{"Switch":{"@odata.id":"/redfish/v1/EthernetSwitches/2"},"PrimaryVLAN":null,"MemberOfPort":null,"PortMembers":[]}}

- c) Submitting  Ethernet port alarms to kafka

ubuntu@voltha-78:~$ kafkacat -C -b localhost:33703 -t voltha.alarms {"category": "OLT", "reported_ts": 1513685248.0, "severity": "CRITICAL", "resource_id": "olt", "id": "voltha.asfvolt16_olt.000139a9a786d85f.olt", "state": "RAISED", "context": {"Health-check-failed-for-Port": "2"}, "type": "EQUIPMENT", "raised_ts": 1513685248.0, "changed_ts": 0.0, "description": "OLT Alarm - Health Monitoring asfvolt16 olt - Raised"}

- System Details

- a) Get System Info

ubuntu@voltha-78:~$ curl -k https://10.70.47.56:8888/redfish/v1/Systems/1 {"@odata.context":"/redfish/v1/$metadata#Systems/Members/$entity","@odata.id":"/redfish/v1/Systems/1","@odata.type":"#ComputerSystem.v1_3_0.ComputerSystem","Id":"1","Name":"Computer System","Description":"Computer System description","SystemType":"Physical","AssetTag":"N/A","Manufacturer":" Accton","Model":" FN1EC0816400Z","SKU":"SKU","SerialNumber":" EC1729003555","PartNumber":" FN1EC0816400Z","UUID":"1db5527c-987d-11e7-ac16-a3de8b31c60c","HostName":null,"Status":{"State":"Enabled","Health":"OK","HealthRollup":"OK"},"IndicatorLED":"Lit","PowerState":"On","BiosVersion":"5.11","ProductName":"ASXvOLT16-O-AC-F","MACAddress":"a8:2b:b5:6d:1c:a5","MACRange":"33","ManufactureDate":"07/31/2017","PlatformName":"x86_64-accton_asxvolt16-r0","LabelRevision":"R0C","CountryCode":"TW","DiagVersion":"0.0.0.3","OnieVersion":"2017.02.00.06","Boot":{"@odata.type":"#ComputerSystem.v1_1_0.Boot","BootSourceOverrideEnabled":"Once","BootSourceOverrideTarget":"Hdd","BootSourceOverrideTarget@Redfish.AllowableValues":["Hdd","Pxe","None"],"BootSourceOverrideMode":"UEFI","BootSourceOverrideMode@Redfish.AllowableValues":["Legacy","UEFI"]},"ProcessorSummary":{"Count":0,"Model":null,"Status":{"State":"Enabled","Health":null,"HealthRollup":null}},"MemorySummary":{"TotalSystemMemoryGiB":null,"Status":{"State":"Enabled","Health":null,"HealthRollup":null}},"Processors":{"@odata.id":"/redfish/v1/Systems/1/Processors"},"EthernetInterfaces":{"@odata.id":"/redfish/v1/Systems/1/EthernetInterfaces"},"NetworkInterfaces":{"@odata.id":"/redfish/v1/Systems/1/NetworkInterfaces"},"Storage":{"@odata.id":"/redfish/v1/Systems/1/Storage"},"Memory":{"@odata.id":"/redfish/v1/Systems/1/Memory"},"PCIeDevices":[],"PCIeFunctions":[],"Links":{"@odata.type":"#ComputerSystem.v1_2_0.Links","Chassis":[{"@odata.id":"/redfish/v1/Chassis/1"}],"Endpoints":[],"ManagedBy":[{"@odata.id":"/redfish/v1/Managers/1"}],"Oem":{}},"Actions":{"#ComputerSystem.Reset":{"target":"/redfish/v1/Systems/1/Actions/ComputerSystem.Reset","ResetType@Redfish.AllowableValues":["ForceOff","GracefulShutdown","GracefulRestart","ForceRestart"]}},"Oem":{"Intel_RackScale":{"@odata.type":"#Intel.Oem.ComputerSystem","PciDevices":[],"PCIeConnectionId":[],"ProcessorSockets":0,"MemorySockets":0,"DiscoveryState":"Basic"}}}

- c) Reboot OLT

ubuntu@voltha-78:~$curl -X POST https://10.70.47.56:8888/redfish/v1/Systems/1/Actions/ComputerSystem.Reset -d '{"ResetType":"GracefulRestart"}' -k